🎮 Full Xbox Controller Control Map (Surface Code V12)

🕹️ Left Joystick

| Input | Axis | Function | Target Motors | Action |
| --- | --- | --- | --- | --- |
| Left Stick Y | 1 | Forward/Backward | M0, M1 | Both same direction (CW = forward) |
| Left Stick X | 0 | Strafe Left/Right | M2, M3 | Both same direction |

🕹️ Right Joystick

| Input | Axis | Function | Target Motors | Action |
| --- | --- | --- | --- | --- |
| Right Stick Y | 3 | Pitch Up/Down | M4, M5 | Opposite direction (M4 CW, M5 CCW = pitch up) |
| Right Stick X | 2 | Roll Left/Right | M2, M3 | Opposite direction (M2 CW, M3 CCW = roll right) |

🎯 Triggers

| Input | Axis | Function | Target Motors | Action |
| --- | --- | --- | --- | --- |
| Right Trigger | 5 | Move Up | M4, M5 | Both CW (same direction) |
| Left Trigger | 4 | Move Down | M4, M5 | Both CCW (same direction) |

🅱️ Buttons

| Button | Index | Function | Action |
| --- | --- | --- | --- |
| LB | 4 | Yaw Left | M0 CW, M1 CCW |
| RB | 5 | Yaw Right | M0 CCW, M1 CW |
| Back | 6 | Exit Joystick Mode | Ends setupJoyStick() loop |

🔘 D-Pad (POV Hat)

| D-Pad Direction | Command Sent | Action |
| --- | --- | --- |
| D-Pad Left | t,angle | Rotate inner arm left by angle |
| D-Pad Right | t,-angle | Rotate inner arm right by angle |
| D-Pad Up | s,1,+angle | Pitch servo up |
| D-Pad Down | s,1,-angle | Pitch servo down |

✋ Buttons A/B/X/Y

| Button | Command Sent | Action |
| --- | --- | --- |
| A | y,-angle | Rotate outer arm in (down) |
| B | y,+angle | Rotate outer arm out (up) |
| X | s,0,+angle | Close claw servo |
| Y | s,0,-angle | Open claw servo |

🛠️ Other Console Commands

| Key Input | Command | Action |
| --- | --- | --- |
| t,angle | Serial from PC | Move inner stepper relative |
| y,angle | Serial from PC | Move outer stepper relative |
| o | Serial from PC | Home inner arm (stepper) |
| y,o | Serial from PC | Home outer arm (stepper) |
| j | Start joystick mode |  |
| q | Quit + close Pico port |  |